# VOLUME 3

# TECHNICAL SPECIFICATIONS

|  |  | UNIT PRICE POSE DESCRIPTIONS |  |
| --- | --- | --- | --- |
|  | Roads to the Heritage Project Construction Work |  |  |
|  | UNIT PRICE POSE DESCRIPTIONS |  |  |
|  | Row Nu | 1 |  |
|  | Pose Nu | KGM/3780/1 |  |
|  | Defination | Roadway railing construction ( RAL - RG 620) |  |
|  | Unit | Mt |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and in accordance with the terms and conditions of the relevant part of the KCT, construction of guardrail tip. Unit Liability Costs Included: 3 mm thick DKP sheet; galvanizing of galvanized residues according to the standard, cleaning of oil and rusts of all materials with chemical bath and cleaning according to the standard, cleaning of galvanized residues by materials, cleaning of materials by project, design of the project, removal of burrs, removal of burrs, removal of burrs, If required by the Inspection Officer, it is necessary to carry out the necessary tests for the control of the galvanizing system, to carry out all the work other than those listed below under the heading "All costs of the workshop or the factory, horizontal and vertical handling, unloading, kind of workmanship, materials, tools and equipment, workshop or factory expenses, contractor profit and general expenses. Units Costs Not Included: Hair; installation, transfer to workshop or factory, evacuation and warehousing. Measure :The length in meters found by multiplying the quantity of the manufactured guardrail in counted quantity by 4 (Four). Payment : In the Unit Price Quotation Exposure is done at unit of KGM / 3780/1 "Guardrail Fleece Construction (RAL-RG 620) (Including Material Price)" |  |
|  | Row Nu | 2 |  |
|  | Pose Nu | KGM/3780/1-M |  |
|  | Defination | Roadway railing assemble ( RAL - RG 620) |  |
|  | Unit | Mt |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and according to the terms and conditions of the relevant part of the KCT, installation of the guardrail in place. Unit Liability Costs Included: Preparing detailed projects for the detection of the location of the guardrail in accordance with the itinerary prepared for the roads determined by the administration, horizontal and vertical movements of the guardrail and the connecting elements at the installation site, disposal, installation in accordance with the project and specifications, tightening of the fasteners all kinds of workmanship, materials, tools and equipment expenses, contractor's profits and general expenses required for the execution of all other works other than those listed under the heading "Costs not included in the unit price" below. Units Costs Not Included: The guardrail rail and its fasteners; loading, unloading and storage. Measure : The length in meters found by multiplying the quantity of the assembled guardrail rail in counted quantity by 4 (Four). Payment : In Unit Price Quotation Exposure Exposure is done at KGM / 3780/1-M "Guardrail Mounting (RAL-RG 620)" unit price |  |
|  | Row Nu | 3 |  |
|  | Pose Nu | KGM/3780/2 |  |
|  | Defination | Roadway railing sewing (Sigma profilden) ( RAL-RG 620) |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and according to the terms and conditions in the relevant part of the KCT, guardrail sewing. Unit Liability Costs Included: 4,2 mm thick DKP sheet; galvanizing of the galvanized residues in accordance with the standard, cleaning of the galvanized residues by the materials, cleaning of the galvanized residues by the materials, cleaning of the galvanized residues according to the standard, Construction Supervisor If necessary, it is necessary to carry out the necessary tests for the control of the galvanizing machinery, all kinds of loading in the workshop or the factory, horizontal and vertical handling, unloading, material loss and all other works other than those listed under the heading " all kind of labor, materials, tools and equipment, workshop or factory expenses, contractor profit and general expenses. Units Costs Not Included: Hair; installation, transfer to workshop or factory, evacuation and warehousing. Measure : The number of pieces of fabricated guardrail sewing is counted. Payment: Unit Price Bidding Chart Exposure Guarding (Sigma Profilden) (RAL-RG 620) (Including Material Price) in KGM / 3780/2 is done on the unit price.. |  |
|  | Row Nu | 4 |  |
|  | Pose Nu | KGM/3780/2-M |  |
|  | Defination | Roadway railing sewing assemble (Sigma profilden) ( RAL-RG 620) |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and according to the terms and conditions in the relevant part of the KCT, guardrail sewing. Unit Liability Costs Included: 4,2 mm thick DKP sheet; galvanizing of the galvanized residues in accordance with the standard, cleaning of the galvanized residues by the materials, cleaning of the galvanized residues by the materials, cleaning of the galvanized residues according to the standard, Construction Supervisor If necessary, it is necessary to carry out the necessary tests for the control of the galvanizing machinery, all kinds of loading in the workshop or the factory, horizontal and vertical handling, unloading, material loss and all other works other than those listed under the heading " all kind of labor, materials, tools and equipment, workshop or factory expenses, contractor profit and general expenses. Units Costs Not Included: Hair; installation, transfer to workshop or factory, evacuation and warehousing. Measure : The number of pieces of fabricated guardrail sewing is counted. Payment: Unit Price Bidding Chart Exposure Guarding (Sigma Profilden) (RAL-RG 620) (Including Material Price) in KGM / 3780/2 is done on the unit price. |  |
|  | Row Nu | 5 |  |
|  | Pose Nu | KGM/3780/4 |  |
|  | Defination | Roadway railing u- block construction ( RAL - RG 620) |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and in accordance with the terms and conditions of the relevant part of the KCT, installation of guardrail plant in place. Unit Liability Costs Included: In accordance with the itinerary prepared for the roads determined by the administration, preparation of detailed projects for the determination of the locations of the guardrail plant, horizontal and vertical movements of the guardrail plant at the installation site, replacement of the guardrail in accordance with the project and specifications, all kinds of labor, materials, tools and supplies necessary for the construction of the excavation, planting and filling of the pit with granular material, removal of excavation residues and all necessary works except for those listed below under the heading "Costs Not Included in Unit Liability" general expenses. Units Costs Not Included: Guardrail stitching and fittings; loading, unloading and storage. Measure: It is the quantity of monted guardrail sewing as the number of pieces. Payment : Unit Price Quotation Exposure is carried out on unit price of KGM / 3780/2-M "Guardrail Mounting (RAL-RG 620) (Sigma Profilden)" unit price. |  |
|  | Row Nu | 6 |  |
|  | Pose Nu | KGM/3780/4-M |  |
|  | Defination | Roadway railing u- block sewing ( RAL - RG 620) |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | Installation of the guardrail in place of the pavement (including the connection plate), in accordance with the project and within the terms and conditions of the relevant part of the PCU. Unit The genius costs: In accordance with the itinerary prepared for the road sections determined by the administration, preparation of detailed projects for the determination of the places of the guardrail, horizontal and vertical movements of the connection plate and connection elements at the installation site, installation in accordance with the projects and specifications, tightening of the connection elements with torque, All costs of labor, materials, tools and equipment, contractor's profits and general expenses required for the execution of all other activities except those listed under the heading "Non-Incidental Costs". Units Costs Not Included: The guardrail block, the connection plate and fittings; loading, unloading and storage. Measure: The number of the assembled guardrails is counted as the number of the pavement (the number of the connection plate is not included). Payment: Exposure in Unit Price Quotation Chart "Guardrail" in KGM / 3780/4-M "Fixture (RAL-RG 620) Mounting" (incl. |  |
|  | Row Nu | 7 |  |
|  | Pose Nu | KGM/3780/10 |  |
|  | Defination | Roadway railing end piece construction (RAL - RG 620) |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and in accordance with the terms and conditions of the relevant part of the KCT, construction of guardrail tip. Unit Liability Costs Included: 3 mm thick DKP sheet; galvanizing of all materials by chemical bath cleaning and hot dipping method in accordance with the standard, removal of galvanized debris from the surface of the galvanized debris, If necessary, it is necessary to carry out the tests for the control of the galvanizing system, to perform all other works except for the ones that are listed under the heading "Costs not included in the unit price" below and all kinds of loading in the workshop or the factory, horizontal and vertical transportation, unloading, material loss. all kind of workmanship, materials, tools and equipment, workshop or factory expenses, contractor profit and general expenses. Units Costs Not Included: Hair; installation, transfer to workshop or factory, evacuation and warehousing. Measure : The manufactured guardrail is the number of pieces of the end piece counted. Payment : In the Unit Price Quotation Exposure KGM / 3780/10 "Guardrail Fleece Construction (RAL-RG 620) (Including Material Price)" unit price. |  |
|  | Row Nu | 8 |  |
|  | Pose Nu | KGM/3780/10-M |  |
|  | Defination | Roadway railing end piece sewing ( RAL - RG 620) |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | Installation of the guardrail end piece in accordance with the project and within the terms and conditions of the relevant part of the WSC. Costs Included in Unit Liability Under the heading "Costs not included in the Unit Costs" below, the preparation of detailed projects to determine the location of the guardrail end pieces in accordance with the itinerary prepared for the determined road sections, horizontal and vertical movements of the end guardrail pieces and fittings at the montage place, all kinds of workmanship, materials, tools and equipment expenses, contractor's profits and general expenses required for the execution of all other activities except those mentioned above. Units Costs Not Included: The guardrail end piece and fittings; loading, unloading, storage. Measure : Mounted guardrail is the number of pieces of the end piece counted. Payment : Unit Price Quotation Exposure The "Guardrail End Mounting (RAL-RG 620)" in KGM / 3780/10-M is done at unit price. |  |
|  | Row Nu | 9 |  |
|  | Pose Nu | KGM/3780/16 |  |
|  | Defination | Construction of simplex reflector for roadway railing |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the project and according to the terms and conditions of the relevant part of the KCT, the construction of a one way reflector for guardrail. Unit Liability Costs Included: 2 mm thick DKP sheet; galvanizing of the galvanized residues by means of hot dipping method in accordance with the standard, cleaning of the oil and rust of all the materials and the standard of the connection elements, cutting of the galvanized residues according to the standards, cutting according to the project and specifications, drawing in the project, drilling of the connection hole according to the project, (Red or White) high performance reflective material in front of the guardrail reflector in accordance with the project of the high performance reflective material, to make the conformity tests of the reflective material if required by the administration, to perform any tests on the workshop or in the factory , horizontal and vertical handling, unloading, material loss, and except for those listed below under the heading "Costs not included in Unit Liability" all kinds of labor, materials, tools and equipment, workshop or factory expenses, contractor's profits and general expenses necessary for the execution of all other works. Units Costs Not Included: With the theme of high performance reflective material; installation, transfer to workshop or factory, evacuation and warehousing. Measure : The manufactured one-way reflector is the quantity in number. Payment : Unit Price Bidding Chart Exposure to KGM / 3780/16 "One Way Reflector for Guardrail (including Material and Connection Elements) (excluding Reflective Materials)" unit price. |  |
|  | Row Nu | 10 |  |
|  | Pose Nu | KGM/3780/16-M |  |
|  | Defination | Assemble of simplex reflector for roadway railing |  |
|  | Unit | Piece |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | Installation of the guardrail reflector in accordance with the project and according to the terms and conditions of the relevant part of the ECHR. Unit Liability Costs Included: In accordance with the itinerary prepared for the road sections determined by the administration, preparation of the detailed projects for the detection of the locations of the guardrail one-way reflector, horizontal and vertical movements of the guardrail one-way reflector and fixing elements, installation in accordance with the projects and specifications, tightening of the connecting elements with torque, All kinds of workmanship, materials, tools and equipment expenses, contractor's profits and general expenses required for the execution of all other works except those listed under the heading "Expenses not included in the prize". Units Costs Not Included: The guardrail reflector and fittings; loading, unloading and storage. Measure : The number of assembled guardrail reflectors is the number of units. Payment : Unit Price Bidding Chart Exposure "Guardrail Reflector Mounting (One Way)" in KGM / 3780/16-M is done by unit price. |  |
|  | Row Nu | 11 |  |
|  | Pose Nu | Y.15.014/4A |  |
|  | Defination | Hard rock excavation without using explosives (free excavation) |  |
|  | Unit | M3 |  |
|  | Book | Ministry of Environment and Urbanisation |  |
|  | Descrition | Hard rocky ground; excavation, loading and unloading of the rocks, unloading of the rocks by the machine without using explosives, dismantling, dismantling, loading on the vehicles, transportation up to 25 meters, storage on the spot or place on the sedan, laying, filling of the gaps left after excavation, 1 m³ excavation price including all kinds of materials and losses, labor, machinery, tools and supplies expenses, contractor general expenses and profits: MEASURE: The excavation volume is calculated on the excavation project. NOTE: 1) This unit does not include fiyata water hike, iksa, transportation outside of 25 meters, full irrigation and compaction charges. 2) Depth is not paid. |  |
|  | Row Nu | 12 |  |
|  | Pose Nu | 12,2206 |  |
|  | Defination | LAYING OF SEWERS PIPE WİTH Ø600 mm ANMA DIEMETER HDPE TYPE 2. |  |
|  | Unit | Mt |  |
|  | Book | Provincial Bank 2006and After Work |  |
|  | Descrition | Providing HDPE-based spiral-wound spiral wound and sewerage pipes manufactured in accordance with TS 12132, all the tests and examinations made before the laying of the HDPE pipes, which are prepared at the side of the trench or where they will be placed, are lowered into the trench with 125 HP Excavator-Beko crane. all kinds of connection elements and miscellaneous parts necessary for the installation of the pipes or pipelines according to the relevant standards, workmanship including all kinds of connection elements and miscellaneous parts, labor, all expenses, tools and equipment expenses, horizontal, vertical transportation, loading (including but not limited to loading, unloading and stacking of this transport with the transport of the pipe from the factory to the beginning of the work), HDPE based pipe laying of spiral wound; 1 meter price. MEASURE: Calculated in meters over the actual length of the furnished pipe. |  |
|  | Row Nu | 13 |  |
|  | Pose Nu | KGM/60.200 |  |
|  | Defination | DRAWING PATHLINE WITH COLD PATHLINE DYEING |  |
|  | Unit | M2 |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | According to the Project, the Traffic Signs Handbook and the drawing of the road lines (by machine) with the cold road marking in accordance with the principles and conditions of the relevant part of the KSD. Unit Liability Costs Included: Construction of the necessary traffic marking at the beginning of the work and protection of this marking during the course of the work, all kinds of studies and preliminary application and technical service works, construction of the road surface to be drawn cleaning, drawing road lines, sprinkling glass syringe automatically on drawn lines, taking precautions to prevent vehicles from passing over the lines until dry, the material used at work; all kinds of labor, materials, machinery, tools and vehicle expenses, contractor's profits and general expenses required for the construction, loading, horizontal and vertical transportation, unloading, all kinds of material loss and all other works except " Unit Liability Costs Included: There is no expense not included in the unit price. Measure: It is the total area in square meters of lines drawn with cold road line paint. Payment: Unit Price Quotation Exposure KGM / 60.200 "Drawing of Road Lines with Cold Road Line (by machine)" m2 unit price. Note: This unit price is applied for each width of the lines up to 0,40 meters. |  |
|  | Row Nu | 14 |  |
|  | Pose Nu | Y.15.001/2B |  |
|  | Defination | Soft and hard soil excavation at every depth and width with the machine (deep excavation) |  |
|  | Unit | M3 |  |
|  | Book | Ministry of Environment and Urbanisation |  |
|  | Descrition | Soft and hard soil; all kinds of materials made for digging and laying of bottom and side walls of excavated area, warehouses and fillings after excavation, laying of excavation by machinery, transporting to vehicles up to 25 meters, unloading of warehouse, and excavation price of 1 m³ including waste, labor, tools and equipment expenses, contractor general expenses and profit: MEASURE: The excavation volume is calculated on the excavation project. NOTE: 1) This unit does not include fiyata water hike, iksa, transportation outside of 25 meters, full irrigation and compaction charges. 2) Depth is not paid. |  |
|  | Row Nu | 15 |  |
|  | Pose Nu | Y.15.006/2B |  |
|  | Defination | Block of loose rock excavation at every depth and width with the machine (deep excavation) |  |
|  | Unit | M3 |  |
|  | Book | Ministry of Environment and Urbanisation |  |
|  | Descrition | On soft and hard hulls; all kinds of materials made for digging and laying of bottom and side walls of excavated area, warehouses and fillings after excavation, laying of excavation by machinery, transporting to vehicles up to 25 meters, unloading of warehouse, and excavation price of 1 m³ including waste, labor, tools and equipment expenses, contractor general expenses and profit: MEASURE: The excavation volume is calculated on the excavation project. NOTE: 1) This unit does not include fiyata water hike, iksa, transportation outside of 25 meters, full irrigation and compaction charges. 2) Depth is not paid. |  |
|  | Row Nu | 16 |  |
|  | Pose Nu | KGM/15.045 |  |
|  | Defination | On all types block of loose rock with the machine |  |
|  | Unit | MT |  |
|  | Book | General Directorate of Highway After 2012 (KGM) |  |
|  | Descrition | The KGM / 2500 numbered position is not included in the Unit Price Quotation Sheet, or the platform (traffic lane, banquette, train, refuge) in each type of crucifixion section of the road segments where the new road axle that has been altered by the Administration leaves more than 25 m from the old road axis specified in the project, , all road width including slopes and ditches, according to the project and according to the terms and conditions of the relevant part of the ICC. Unit Liability Costs Included: All kinds of workmanship required for the construction of the road platform, the ditches, the filling and splitting slope surfaces of the appropriate machinery and, if necessary, the fine finishing of the work, the embossing of the floor at a depth of 20 cm on the floor and the following exceptions under the heading "Costs not included in the unit price" , material, machinery, tools and vehicle expenses and contractor profits and general expenses. Units Costs Not Included: Where appropriate, appropriate materials supplied from the yarn and / or from the yarn; all irrigation and compaction works to be carried out. Measure : Roads or miles along the common road axis (two or more road axes), which are completed all the way broadly acceptable, including slopes, in accordance with the cross-sections, curves, slopes and elevations shown in the projects and the directive of the Building Inspection Officer fractions of fractions. No separate measure is made for each road lane. Separate measurements shall be made in the divided roads for the departure and return roads separated by more than 25 m from the distance between the road axes. Note: (1) Base regulation price Fb = 13 km for platform width 1 km for a road section with platform width (a) m for the regulation of the entire surface (slope, ditch, platform) between the outer edges of the slope Fa = Fbx 0,35 + 0,05xa). (2) To account for platform width in divided ways with joint axes; northern islands and northern islands will not be included. (3) Even if the road axis specified in the project has changed for any reason, this unit price is applied exactly. |  |
|  | Row Nu | 17 |  |
|  | Pose Nu | KGM/60.220 |  |
|  | Defination | STANDARD ARROW AND WRITING YAZILARIN WITH PATHLINE DYEING |  |
|  | Unit | Piece |  |
|  | Book | Ministry of Environment and Urbanisation |  |
|  | Descrition | According to the Project, the Traffic Signs Handbook and the standard lines and arrows with cold road marking under the terms and conditions of the relevant part of the KTŞ. Unit Liability Costs Included: Protecting this marking during the course of the work, cleaning the place to be drawn, drawing standard arrows, symbols and letters according to the project, sprinkling of glass syrup on the drawn lines, painting of the paint, glass syringe and the template at work, transportation of the line work to the work, , all kinds of workmanship, materials, machinery, tools and vehicle expenses and contractor profits required for the construction of all other activities except for the following: "taking measures to prevent the vehicles from passing through to dry, technical services, all kind of material losses and" and general expenses. Units Costs Not Included: There is no expense not included in the unit price. Measure: The drawn standard is the name of the arrow, symbol, number and letters. Payment: Unit Price Quote Schedules Exposure is done on unit price of KGM / 60.220 "Drawing Standard Lines and Standard Arrows". |  |
|  | Row Nu | 18 |  |
|  | Pose Nu | SPECIAL POSE |  |
|  | Defination | CONSTRUCTION OF A SHEET ASPAHLT SURFACE TREATMENT WITH BITUMINOUS (WITH CRUSHED AND SIFTED 19.0 - 12.5 MM HEARTSTONE) (CONSTRUCTION OF SUB- BASE AND INCLUDING SHIPPING) (5200m X 8,00 m = 41600 m²) |  |
|  | Unit | M2 |  |
|  | Book | PROJECT SPECIAL |  |
|  | Descrition | Basic construction with 25 mm (I ") material from quarry, crusher and crusher in accordance with the requirements and conditions in the relevant part of the KSŞ. Unit Liability Costs Included: Move; dismantling with crushing and crushing with crusher to obtain the granulometry specified in KCT, loading and unloading of carcass, loading and unloading of carcasses, all kinds of workmanship, materials, machinery, tools and tools necessary for the construction of the water with the motor pump, the laying and compression of the basic material by providing the optimum water content and the following "All the other activities which are not included in the unit cost" and contractor profits and overheads. Note: (1) The figure shall be in the place, shape and size to be requested by the administrator and shall be done with the machine of the figure. (2) The contractor shall preserve the figure and shall take out the depressions and defects. According to the project and in accordance with the terms and conditions of the relevant part of the KSŞ, a layer-slipped bituminous surface coating with crushed and screened quarry (Type-2). Unit Liability Costs Included: Required machinery and equipment; installation, disassembly, stone; the crushing and sifting of the crusher with the crusher to obtain the desired granulometry specified in the specifications, the discharge of the crusher, the loading of the aggressor, the loading and unloading of the aggregate, the loading and unloading of the crusher, the loading and unloading of the crusher, weighing with weighers, at least two of the road surface before the primer, at least once after the primer, all the widths and sweeping of the joints at the time of the coating and throwing out the dregs from the platform, the tank tanks, cistern wagons, tanker and roley tanks material; transfer to the storage tanks, storage and use up to the distributor, transfer to the distributor with asphalt pump, heating up to the application level, priming of the road surface by spraying with the spray pipe or where the pipe is not approaching, heating the binder bitumen material to the extent of the absorption in the tank tank and transferring it to the distributor. the cargo trucks are waiting until they reach their turn, waiting for the gravel to drain, laying the grate on the ground, laying the carpets on the places where the cargo can not reach, hand rolling with the tire wheel cylinders, sweeping the freewheel on the road surface by machine, in the sweeping works done in the city, the loading and unloading of the trash material placed on the workbench, transportation and bo inert, be, by making the research and technical supervision work following "Unit Price excludes Charges" all kinds required to be done, except from all the other things labor, materials, machinery, contractor with tools and tool expenses profit and general expenses. Note: (1) The contractor has to manufacture according to the design given to the Administration; there is no change in unit price due to design change. (2) The written consent of the address and the distance between the hearth and the crane; if the final average is less than 150 m5 d and less than 10 000 m, the carriage distance is (M) meters: F = A x 1,25 x 0,00017 x K x M½ - 0,00260 x K TL / ton is calculated and paid. In this form (M½), (K), (A) and (Y) are the same as Exposure 07.005 / K. (3) In the case that the distance between the written consent and the quarry-crusher is more than 10 000 m9, the carriage distance is (M) miles: F = A x 1,25 x K x (0,0007 x M + 0,01) - 0,00260 x K TL / ton is calculated and paid. In this formula (M), (K), (A) and (Y), the exposure is like 07,006 / K. |  |

|  |  | BİRİM FİYAT POZ TARİFLERİ |  |
| --- | --- | --- | --- |
|  | Sarpdere Köyü Dupnisa Mağarası Bisiklet Yolu İnşaatı |  |  |
|  | PROJE BIRIM FIYAT POZ TARİFLERİ |  |  |
|  | Sıra No | 1 |  |
|  | Poz No | KGM/3780/1 |  |
|  | Tanım | OTOKORKULUK RAYI YAPILMASI |  |
|  | Birim | MT |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk rayı yapılması.Birim Fiyata Dahil Olan Masraflar :3 mm kalınlıktaki DKP sacın; bedeli, projesine ve şartnamesine göre kesilmesi, projesindeki şekle getirilmesi, bağlantı deliklerinin delinmesi, çapakların giderilmesi, gerekli bağlantı elemanlarının temini, bütün malzemelerin yağ ve paslarının kimyasal banyo ile temizlenmesi ve standardına uygun olarak sıcak daldırma usulü ile galvanizlenmesi, galvaniz artıklarının malzemeler üzerinden temizlenmesi, Yapı Denetim Görevlisince gerekli görülmesi halinde galvanizlemenin kontrolü için gerekli deneylerin yaptırılması, atölye veya fabrikadaki her türlü yükleme, yatay ve düşey taşıma, boşaltma, malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç ile atölye veya fabrika giderleri, yüklenici karı ve genel masraflar.Birim Fiyata Dahil Olmayan Masraflar :Sacın; yüklenmesi, atölye veya fabrikaya taşınması, boşaltılması ve istifi. Ölçü : İmal edilmiş otokorkuluk rayının sayılan adet olarak miktarının 4 (Dört) ile çarpılması suretiyle bulunan metre cinsinden uzunluğudur. Ödeme:Birim Fiyat Teklif Cetvelinde Poz KGM/3780/1 deki "Otokorkuluk Rayı Yapılması (RAL-RG 620) (Malzeme Bedeli Dahil)" m birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 2 |  |
|  | Poz No | KGM/3780/1-M |  |
|  | Tanım | OTOKORKULUK RAYI MONTAJI |  |
|  | Birim | Mt |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk rayının yerine montajı. Birim Fiyata Dahil Olan Masraflar : İdarece belirlenen yol kesimleri için hazırlanan itinerere uygun olarak otokorkuluk rayının yerlerinin saptanması için detay projelerin hazırlanması, otokorkuluk rayının ve bağlantı elemanlarının montaj yerinde yatay ve düşey taşınması, boşaltılması, proje ve şartnamesine uygun olarak yerine montajı, bağlantı elemanlarının torkunda sıkılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Otokorkuluk rayının ve bağlantı elemanlarının; yüklenmesi, montaj yerine taşınması, boşaltılması ve istifi. Ölçü : Monte edilmiş otokorkuluk rayının sayılan adet olarak miktarının 4 (Dört) ile çarpılması suretiyle bulunan metre cinsinden uzunluğudur. Ödeme : Birim Fiyat Teklif Cetvelinde Poz KGM/3780/1-M deki "Otokorkuluk Rayı Montajı (RAL-RG 620)" m birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 3 |  |
|  | Poz No | KGM/3780/2 |  |
|  | Tanım | OTOKORKULUK DİKMESİ YAPILMASI |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk dikmesi yapılması. Birim Fiyata Dahil Olan Masraflar : 4,2mm kalınlıktaki DKP sacın; bedeli, projesine ve şartnamesine göre kesilmesi, projesindeki şekle getirilmesi, bağlantı deliklerinin delinmesi, çapakların giderilmesi, gerekli olan bağlantı elemanlarının temini, bütün malzemelerin yağ ve paslarının kimyasal banyo ile temizlenmesi, standardına uygun olarak sıcak daldırma usulü ile galvanizlenmesi, galvaniz artıklarının malzemeler üzerinden temizlenmesi, Yapı Denetim Görevlisince gerekli görülmesi halinde galvanizlemenin kontrolü için gerekli deneylerin yaptırılması, atölye veya fabrikadaki her türlü yükleme, yatay ve düşey taşıma, boşaltma, malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç ile atölye veya fabrika giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Sacın; yüklenmesi, atölye veya fabrikaya taşınması, boşaltılması ve istifi. Ölçü : İmal edilmiş otokorkuluk dikmesinin sayılan adet olarak miktarıdır. Ödeme: Birim Fiyat Teklif Cetvelinde Poz KGM/3780/2 deki "Otokorkuluk Dikmesi Yapılması (Sigma Profilden) (RAL-RG 620) (Malzeme Bedeli Dahil)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 4 |  |
|  | Poz No | KGM/3780/2-M |  |
|  | Tanım | OTOKORKULUK DİKMESİ MONTAJI |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk dikmesinin yerine montajı. Birim Fiyata Dahil Olan Masraflar : İdarece belirlenen yol kesimleri için hazırlanan itinerere uygun olarak otokorkuluk dikmesinin yerlerinin saptanması için detay projelerin hazırlanması, otokorkuluk dikmesinin montaj yerinde yatay ve düşey taşınması, proje ve şartnamesine uygun olarak yerine çakılması, zemin şartları nedeniyle gereken derinlikte çakılamayan durumda Yapı Denetim Görevlisinin izni ile gerekli derinlikte çukur kazılması, dikmenin yerleştirilmesi ve çukurun granüler malzeme ile doldurularak sıkıştırılması, kazı artıklarının uzaklaştırılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan gerekli bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Otokorkuluk dikmesinin ve bağlantı elemanlarının; yüklenmesi, montaj yerine taşınması, boşaltılması ve istifi. Ölçü: Monte edilmiş otokorkuluk dikmesinin sayılan adet olarak miktarıdır. Ödeme : Birim Fiyat Teklif Cetvelinde Poz KGM/3780/2-M deki "Otokorkuluk Dikmesi Montajı (RAL-RG 620) (Sigma Profilden)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 5 |  |
|  | Poz No | KGM/3780/4 |  |
|  | Tanım | OTOKORKULUK "U" TAKOZU YAPILMASI |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk U takozu yapılması. Birim Fiyata Dahil Olan Masraflar : 5 mm kalınlıktaki DKP sacın; bedeli, projesine ve şartnamesine göre kesilmesi, projesindeki şekle getirilmesi, bağlantı deliğinin delinmesi, çapakların giderilmesi, gerekli bağlantı elemanlarının temini, bütün malzemelerin yağ ve paslarının kimyasal banyo ile temizlenmesi ve standardına uygun olarak sıcak daldırma usulü ile galvanizlenmesi, galvaniz artıklarının malzemeler üzerinden temizlenmesi, İdarece gerekli görülmesi halinde galvanizlemenin kontrolü için deneylerin yaptırılması, atölye veya fabrikadaki her türlü yükleme, yatay ve düşey taşıma, boşaltma, malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç ile atölye veya fabrika giderleri, yüklenici kan ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Sacın; yüklenmesi, atölye veya fabrikaya taşınması, boşaltılması ve istifi. Ölçü : İmal edilmiş otokorkuluk U takozunun sayılan adet olarak miktarıdır. Ödeme: Birim Fiyat Teklif Cetvelinde Poz KGM/3780/4 deki "Otokorkuluk "U" Takozu Yapılması (RAL-RG 620) (Malzeme Bedeli Dahil)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 6 |  |
|  | Poz No | KGM/3780/4-M |  |
|  | Tanım | OTOKORKULUK ''U'' TAKOZU MONTAJI |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk U takozunun (bağlantı plakası dahil) yerine montajı. Birim Fiyata Dahi Olan Masraflar: İdarece belirlenen yol kesimleri için hazırlanan itinerere uygun olarak otokorkuluk U takozunun yerlerinin saptanması için detay projelerin hazırlanması, takozun, bağlantı plakasının ve bağlantı elemanlarının montaj yerinde yatay ve düşey taşınması, proje ve şartnamesine uygun olarak yerine montajı, bağlantı elemanlarının torkunda sıkılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Otokorkuluk U takozunun, bağlantı plakasının ve bağlantı elemanlarının; yüklenmesi, montaj yerine taşınması, boşaltılması ve istifi. Ölçü: Monte edilmiş otokorkuluk U takozunun sayılan adet olarak miktarıdır (Bağlantı plakası sayıya dahil edilmez). Ödeme: Birim Fiyat Teklif Cetvelinde Poz KGM/3780/4-M deki "Otokorkuluk "U" Takozu (Bağlantı Plakası Dahil) Montajı (RAL-RG 620)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 7 |  |
|  | Poz No | KGM/3780/10 |  |
|  | Tanım | OTOKORKULUK UÇ PARÇASI YAPILMASI |  |
|  | Birim | Adet |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk uç parçası yapılması. Birim Fiyata Dahil Olan Masraflar : 3 mm kalınlıktaki DKP sacın; bedeli, projesine ve şartnamesine göre kesilmesi, projesindeki şekle getirilmesi, bağlantı deliklerinin delinmesi, gerekli kaynak işlerinin yapılması, çapakların giderilmesi, gerekli bağlantı elemanlarının temini, bütün malzemelerin yağ ve paslarının kimyasal banyo ile temizlenmesi ve standardına uygun olarak sıcak daldırma usulü ile galvanizlenmesi, galvaniz artıklarının malzemeler üzerinden temizlenmesi, İdarece gerekli görülmesi halinde galvanizlemenin kontrolü için deneylerin yaptırılması, atölye veya fabrikadaki her türlü yükleme, yatay ve düşey taşıma, boşaltma, malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç ile atölye veya fabrika giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Sacın; yüklenmesi, atölye veya fabrikaya taşınması, boşaltılması ve istifi. Ölçü : İmal edilmiş otokorkuluk uç parçasının sayılan adet olarak miktarıdır. Ödeme : Birim Fiyat Teklif Cetvelinde Poz KGM/3780/10 daki "Otokorkuluk Uç Parçası Yapılması (RAL-RG 620) (Malzeme Bedeli Dahil)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 8 |  |
|  | Poz No | KGM/3780/10-M |  |
|  | Tanım | OTOKORKULUK UÇ PARÇASI MONTAJI |  |
|  | Birim | Adet |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk uç parçasının yerine montajı. Birim Fiyata Dahil Olan Masraflar İdarece belirlenen yol kesimleri için hazırlanan itinerere uygun olarak otokorkuluk uç parçasının yerlerinin saptanması için detay projelerin hazırlanması, otokorkuluk uç parçasının ve bağlantı elemanlarının montaj yerinde yatay ve düşey taşınması, yerine montajı, bağlantı elemanlarının torkunda sıkılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Otokorkuluk uç parçasının ve bağlantı elemanlarının; yüklenmesi, montaj yerine taşınması, boşaltılması, istifi. Ölçü : Monte edilmiş otokorkuluk uç parçasının sayılan adet olarak miktarıdır. Ödeme : Birim Fiyat Teklif Cetvelinde Poz KGM/3780/10-M deki "Otokorkuluk Uç Parçası Montajı (RAL-RG 620)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 9 |  |
|  | Poz No | KGM/3780/16 |  |
|  | Tanım | OTOKORKULUK İÇİN TEK YÖNLÜ REFLEKTÖR YAPILMASI |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk için tek yönlü reflektör yapılması. Birim Fiyata Dahil Olan Masraflar : 2mm kalınlıktaki DKP sacın; bedeli, projesine ve şartnamesine göre kesilmesi, projesindeki şekle getirilmesi, bağlantı deliğinin projesine uygun olarak delinmesi, çapakların giderilmesi, gerekli bağlantı elemanlarının temini, bütün malzemelerin yağ ve paslarının kimyasal banyo ile temizlenmesi ve standardına uygun olarak sıcak daldırma usulü ile galvanizlenmesi, galvaniz artıklarının malzemeler üzerinden temizlenmesi, İdarece gerekli görülmesi halinde galvanizlemenin kontrolü için deneylerin yaptırılması, otokorkuluk reflektörünün ön tarafına İdarece istenilen renkte (Kırmızı veya Beyaz) yüksek performanslı reflektif malzemenin projesine göre kesilmesi ve yapıştırılması, İdarece gerekli görülmesi halinde reflektif malzemenin uygunluk deneylerinin yaptırılması, atölye veya fabrikadaki her türlü yükleme, yatay ve düşey taşıma, boşaltma, malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç ile atölye veya fabrika giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Yüksek performanslı reflektif malzemenin temini ile sacın; yüklenmesi, atölye veya fabrikaya taşınması, boşaltılması ve istifi. Ölçü : İmal edilmiş tek yönlü reflektörün sayılan adet olarak miktarıdır. Ödeme : Birim Fiyat Teklif Cetvelinde Poz KGM/3780/16 daki "Otokorkuluk İçin Tek Yönlü Reflektör Yapılması (Malzeme Bedeli ve Bağlantı Elemanı Dahil) (Reflektif Malzeme Hariç)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 10 |  |
|  | Poz No | KGM/3780/16-M |  |
|  | Tanım | OTOKORKULUK REFLEKTÖRÜ MONTAJI (TEK YÖNLÜ) |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, otokorkuluk reflektörünün yerine montajı. Birim Fiyata Dahil Olan Masraflar : İdarece belirlenen yol kesimleri için hazırlanan itinerere uygun olarak otokorkuluk tek yönlü reflektörünün yerlerinin saptanması için detay projelerin hazırlanması, otokorkuluk tek yönlü reflektörünün ve bağlantı elemanlarının montaj yerinde yatay ve düşey taşınması, proje ve şartnamesine uygun olarak yerine montajı, bağlantı elemanlarının torkunda sıkılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar" başlığı altında sayılanlar dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, araç ve gereç giderleri, yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar : Otokorkuluk reflektörünün ve bağlantı elemanlarının; yüklenmesi, montaj yerine taşınması, boşaltılması ve istifi. Ölçü : Monte edilmiş otokorkuluk reflektörünün sayılan adet olarak miktarıdır. Ödeme : Birim Fiyat Teklif Cetvelinde Poz KGM/3780/16-M deki "Otokorkuluk Reflektörü Montajı (Tek Yönlü)" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 11 |  |
|  | Poz No | Y.15.014/4A |  |
|  | Tanım | Makine ile patlayıcı madde kullanmadan sert kaya kazılması (serbest kazı) |  |
|  | Birim | M3 |  |
|  | Kitap | Çevre ve Şehircilik Bakanlığı (BAY) |  |
|  | Tarifi | Sert kayalık zeminde; patlayıcı madde kullanılmadan makina ile kayaların kırılması, sökülmesi, kazılması, taşıtlara yüklenmesi, 25 metreye kadar taşınması, depo, imla veya sedde yerinde boşaltılması, serilmesi, inşaat yapıldıktan sonra kazı yerinde kalan boşlukların doldurulması, kazı yeri, depo ve dolgunun tesviyesi ve düzeltilmesi için yapılan her türlü malzeme ve zayiatı, işçilik, makine, araç ve gereç giderleri, müteahhit genel giderleri ve kârı dâhil, 1 m³ kazı fiyatı: ÖLÇÜ: Kazının hacmi kazı projesi üzerinden hesaplanır. NOT: 1)Bu birim fiyata su zammı, iksa, 25 metre dışındaki taşıma, dolgunun sulama ve sıkıştırma bedelleri dâhil değildir. 2)Derinlik zammı ödenmez.2) Depth is not paid. |  |
|  | Sıra No | 12 |  |
|  | Poz No | 12,2206 |  |
|  | Tanım | Ø600 mm ANMA ÇAPLI HDPE KANALİZASYON BORUSUNUN DÖŞENMESİ TİP 2. |  |
|  | Birim | MT |  |
|  | Kitap | İller Bankası 2006 ve Sonrası İşler (İLC) |  |
|  | Tarifi | TS 12132 'e uygun imal edilen HDPE esaslı spiral sarımlı yağmursuyu ve kanalizasyon borularının temin edilmesi, döşenmeden önce her türlü deneyi ve muayenesi yapılmış olarak hendek kenarında veya konulacağı yerde hazırlanmış, HDPE boruların 125 HP Ekskavatör-Beko vinç ile hendek içerisine indirilmesi, şartnamesine ve ilgili standartlarına göre yerleştirilmesi, döşenmiş boru başlarının veya boru hattının ilgili standartlara göre sızdırmazlık tecrübesinin yapılması için, gerekli her türlü bağlantı elemanı ve müteferrik parçaları ile malzeme ve zayiatı, deney dahil işçilik, her türlü masraf, araç ve gereç giderleri, yatay, düşey taşıma, yükleme, boşaltma ile müteahhit karı ve genel giderler dahil (Yalnız, borunun fabrikadan iş başına kadar taşınması ile bu taşımaya ait yükleme, boşaltma ve istif bedeli hariç) spiral sarımlı HDPE esaslı boru döşenmesinin; 1 metre fiyatı. ÖLÇÜ : Döşenmiş haldeki borunun gerçek uzunluğu üzerinden metre olarak hesaplanır. |  |
|  | Sıra No | 13 |  |
|  | Poz No | KGM/60.200 |  |
|  | Tanım | SOĞUK YOL ÇİZGİ BOYASI İLE YOL ÇİZGİLERİNİN ÇİZİLMESİ |  |
|  | Birim | M2 |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine, Trafik İşaretleri El Kitabına göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, soğuk yol çizgi boyası ile yol çizgilerinin çizilmesi (makine ile). Birim Fiyata Dahil Olan Masraflar : Çizgi ve süpürge makinesi, soğuk yol çizgi boyası, cam küreciği, etüt ile çizgi ekibinin iş başında temini, çalışma başlangıcında gerekli trafik işaretlemesinin yapılması ve işin devamı süresince bu işaretlemenin korunması, her türlü etüt ve ön aplikasyon ile teknik hizmet işlerinin yapılması, çizilecek yol sathının temizlenmesi, yol çizgilerin çizilmesi, çizilen çizgiler üzerine otomatik olarak cam küreciği serpilmesi, çizgiler kuruyuncaya kadar üzerinden taşıtların geçmesini önleyecek tedbirlerin alınması, iş yerinde kullanılan malzemenin; yüklenmesi, yatay ve düşey taşınması, boşaltılması, her türlü malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar"dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, makine, alet ve araç giderleri ile yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olan Masraflar: Birim fiyata dahil olmayan masraf yoktur. Ölçü: Soğuk yol çizgi boyası ile çizilen çizgilerin metrekare cinsinden toplam alanıdır. Ödeme: Birim Fiyat Teklif Cetvelinde Poz KGM/60.200 deki "Soğuk Yol Çizgi Boyası İle Yol Çizgilerinin Çizilmesi (Makine ile)" m2 birim fiyatı üzerinden yapılır. Not: Bu birim fiyat 0,40 metreye kadar her genişlikteki çizgiler için uygulanır. |  |
|  | Sıra No | 14 |  |
|  | Poz No | Y.15.001/2B |  |
|  | Tanım | Makine ile her derinlik ve her genişlikte yumuşak ve sert küskülük kazılması (derin kazı) |  |
|  | Birim | M3 |  |
|  | Kitap | Çevre ve Şehircilik Bakanlığı (BAY) |  |
|  | Tarifi | Yumuşak ve sert küskülük zeminde; makina ile kazının yapılması, taşıtlara yüklenmesi, 25 metreye kadar taşınması, depo, imla veya sedde yerinde boşaltılması, serilmesi, inşaat yapıldıktan sonra kazı yerinde kalan boşlukların doldurulması, kazılan yerin taban ve yan cidarlarının, depo ve dolgunun tesviyesi ve düzeltilmesi için yapılan her türlü malzeme ve zayiatı, işçilik, araç ve gereç giderleri, müteahhit genel giderleri ve kârı dâhil, 1 m³ kazı fiyatı: ÖLÇÜ: Kazının hacmi kazı projesi üzerinden hesaplanır. NOT: 1)Bu birim fiyata su zammı, iksa, 25 metre dışındaki taşıma, dolgunun sulama ve sıkıştırma bedelleri dâhil değildir. 2)Derinlik zammı ödenmez. |  |
|  | Sıra No | 15 |  |
|  | Poz No | Y.15.006/2B |  |
|  | Tanım | Makine ile her derinlik ve her genişlikte yumuşak ve sert küskülük kazılması (derin kazı) |  |
|  | Birim | M3 |  |
|  | Kitap | Çevre ve Şehircilik Bakanlığı (BAY) |  |
|  | Tarifi | Yumuşak ve sert küskülük zeminde; makina ile kazının yapılması, taşıtlara yüklenmesi, 25 metreye kadar taşınması, depo, imla veya sedde yerinde boşaltılması, serilmesi, inşaat yapıldıktan sonra kazı yerinde kalan boşlukların doldurulması, kazılan yerin taban ve yan cidarlarının, depo ve dolgunun tesviyesi ve düzeltilmesi için yapılan her türlü malzeme ve zayiatı, işçilik, araç ve gereç giderleri, müteahhit genel giderleri ve kârı dâhil, 1 m³ kazı fiyatı: ÖLÇÜ: Kazının hacmi kazı projesi üzerinden hesaplanır. NOT: 1)Bu birim fiyata su zammı, iksa, 25 metre dışındaki taşıma, dolgunun sulama ve sıkıştırma bedelleri dâhil değildir. 2)Derinlik zammı ödenmez. |  |
|  | Sıra No | 16 |  |
|  | Poz No | KGM/15.045 |  |
|  | Tanım | On all types block of loose rock with the machine |  |
|  | Birim | KM |  |
|  | Kitap | Ministry of Environment and Urbanisation |  |
|  | Tarifi | The KGM / 2500 numbered position is not included in the Unit Price Quotation Sheet, or the platform (traffic lane, banquette, train, refuge) in each type of crucifixion section of the road segments where the new road axle that has been altered by the Administration leaves more than 25 m from the old road axis specified in the project, , all road width including slopes and ditches, according to the project and according to the terms and conditions of the relevant part of the ICC. Unit Liability Costs Included: All kinds of workmanship required for the construction of the road platform, the ditches, the filling and splitting slope surfaces of the appropriate machinery and, if necessary, the fine finishing of the work, the embossing of the floor at a depth of 20 cm on the floor and the following exceptions under the heading "Costs not included in the unit price" , material, machinery, tools and vehicle expenses and contractor profits and general expenses. Units Costs Not Included: Where appropriate, appropriate materials supplied from the yarn and / or from the yarn; all irrigation and compaction works to be carried out. Measure : Roads or miles along the common road axis (two or more road axes), which are completed all the way broadly acceptable, including slopes, in accordance with the cross-sections, curves, slopes and elevations shown in the projects and the directive of the Building Inspection Officer fractions of fractions. No separate measure is made for each road lane. Separate measurements shall be made in the divided roads for the departure and return roads separated by more than 25 m from the distance between the road axes. Note: (1) Base regulation price Fb = 13 km for platform width 1 km for a road section with platform width (a) m for the regulation of the entire surface (slope, ditch, platform) between the outer edges of the slope Fa = Fbx 0,35 + 0,05xa). (2) To account for platform width in divided ways with joint axes; northern islands and northern islands will not be included. (3) Even if the road axis specified in the project has changed for any reason, this unit price is applied exactly. |  |
|  | Sıra No | 17 |  |
|  | Poz No | KGM/60.220 |  |
|  | Tanım | SOĞUK YOL ÇİZGİ BOYASI İLE STANDART OK VE YAZILARIN ÇİZİLMESİ |  |
|  | Birim | AD |  |
|  | Kitap | Karayolları Genel Müdürlüğü 2012 Sonrası (KGM) |  |
|  | Tarifi | Projesine, Trafik İşaretleri El Kitabına göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, soğuk yol çizgi boyası ile standart ok ve yazıların çizilmesi. Birim Fiyata Dahil Olan Masraflar: Boya, cam küreciği ve şablonun iş başında temini, çizgi ekibinin iş başına taşınması, çalışma başlangıcında gerekli trafik işaretlemesinin yapılması ve işin devamı süresince bu işaretlemenin korunması, çizilecek yerin temizlenmesi, projesine göre standart ok, sembol ve yazıların çizilmesi, çizilen çizgiler üzerine cam küreciği serpilmesi, kuruyuncaya kadar üzerinden taşıtların geçmesini önleyecek tedbirlerin alınması, teknik hizmetler, her türlü malzeme zayiatı ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar"dışmda kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, makine, alet ve araç giderleri ile yüklenici karı ve genel masraflar. Birim Fiyata Dahil Olmayan Masraflar: Birim fiyata dahil olmayan masraf yoktur. Ölçü: Çizilen standart ok,sembol,rakam ve harflerin sayılan adedidir. Ödeme: Birim Fiyat Teklif Cetvelinde Poz KGM/60.220 deki "Soğuk Yol Çizgi Boyası ile Standart Ok ve Yazıların Çizilmesi" adet birim fiyatı üzerinden yapılır. |  |
|  | Sıra No | 18 |  |
|  | Poz No | Özel Poz |  |
|  | Tanım | BİR TABAKA ASTARLI BİTÜMLÜ SATHİ KAPLAMA YAPILMASI(KIRILMIŞ VE ELENMİŞ 19.0 - 12.5 MM OCAK TAŞI İLE)(ALTTEMEL YAPILMASI VE NAKLİYELER DAHİL) (5200m x 8,00 m =41600,00 m²) |  |
|  | Birim | M2 |  |
|  | Kitap | Piyasa Fiyat Araştırması |  |
|  | Tarifi | KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, ocak taşından, konkasörle kırılmış ve elenmiş 25 mm (I")' lik malzeme ile temel yapılması. Birim Fiyata Dahil Olan Masraflar: Taşın; ocaktan çıkarılması, konkasöre verilecek boyutta kırılması, vasıtalara yüklenmesi, ocak - konkasör arasında nihai ortalama 150 m mesafeye kadar taşınması, boşaltılması, konkasöre verilmesi, granülometri ve evsaf araştırması yapılması, KTŞ' de belirtilen granülometriyi elde edecek şekilde konkasörle kırılması, elenmesi, vasıtalara yüklenmesi, boşaltılması ve figüre edilmesi, motorlu tulumba ile suyun hazırlanması, temel malzemesinin optimum su içeriği sağlanarak serilmesi ve sıkıştırılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflaf'dışmda kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, makine, alet ve araç giderleri ile yüklenici karı ve genel masraflar. Not: (1)Figüre İdarece istenilecek yer, şekil ve boyutta olacak ve figüre makinesi ile yapılacaktır. (2)Yüklenici figüreyi muhafaza edecek, çöküntü ve bozuklukları giderecektir. Projesine göre ve KTŞ'nin ilgili kısmındaki esaslar ve şartlar dahilinde, kırılmış ve elenmiş ocak taşı ile bir tabaka astarlı bitümlü sathi kaplama yapılması (Tip-2). Birim Fiyata Dahil Olan Masraflar : Gerekli makine ve teçhizatın; iş başında temini, montajı ve demontajı, taşın; ocaktan çıkarılması, konkasöre verilecek ebatta kırılması, vasıtalara yüklenmesi, ocak-konkasör arasında nihai ortalama 150 m mesafeye kadar taşınması, boşaltılması, konkasöre verilmesi, şartnamesinde belirtilen veya İdarece istenilen granülometriyi elde edecek şekilde konkasörle kırılması ve elenmesi, konkasör altının boşaltılması, agreganın vasıtalara yüklenmesi, kantarla tartılması, yol yüzeyinin astardan evvel en az iki, astardan sonra en az bir defa olmak üzere bütün genişliğin ve kaplama anında ek yerlerinin süpürülmesi ve çıkan süprüntü malzemesinin platform dışına atılması, depo tankının temini, sarnıçlı vagon, tanker ve roley tank gibi kaplarla taşman bitümlü malzemenin; depo tanklarına aktarılması, depolanması ve kullanılıncaya kadar muhafazası, asfalt pompası ile distribütöre aktarılması, tatbik derecesine kadar ısıtılması, püskürtme borusu ile veya borunun yanaşmadığı yerlerde elle püskürtülmesi suretiyle yol yüzeyinin astarlanması, bağlayıcı bitümlü malzemenin depo tankında emiş derecesine kadar ısıtılması ve distribütöre aktarılması, tatbik derecesine kadar ısıtılması ve püskürtülmesi, taşıma kamyonlarının kendilerine sıra gelinceye kadar mıcır sericiyi beklemesi, mıcırın sericiye boşaltılması, serilmesi, sericinin yanaşamadığı yerlere mıcırın el ile serilmesi, lastik tekerlekli silindirle sıkıştırılması, kaplama sonrasında yol yüzeyindeki serbest malzemenin makine ile süpürülmesi, çıkan malzemenin platform dışına atılması, şehir içinde yapılan süpürme işlerinde tretuvar üzerine konulan süprüntü malzemesinin vasıtalara yüklenmesi, taşınması ve boşaltılması, araştırma ve teknik nezaret işlerinin yapılması ile aşağıda "Birim Fiyata Dahil Olmayan Masraflar"dışında kalan diğer bütün işlerin yapılması için gerekli olan her türlü işçilik, malzeme, makine, alet ve araç giderleri ile yüklenici karı ve genel masraflar. Not: (1)Yüklenici, İdarece verilen dizayna göre imalat yapmak zorunda olduğundan; dizayn değişmesinden dolayı birim fiyatta herhangi bir değişiklik yapılmaz. (2)İdarenin yazılı izni ile ocak-konkasör arasındaki mesafenin; nihai ortalama 150 m5 den fazla ve 10 000 m'den az olması halinde, (M) metre mesafeye taşıma bedeli: F = A x 1,25 x 0,00017 x K x M½ - 0,00260 x K TL/ton formülü ile hesaplanarak ödenir. Bu formüldeki (M½), (K), (A) ve (Y), Poz 07.005/K daki gibidir. (3)İdarenin yazılı izni ile ocak-konkasör arasındaki mesafenin 10 000 m9den fazla olması halinde, bu (M) kilometre mesafeye taşıma bedeli: F = A x 1,25 x K x (0,0007 x M + 0,01) - 0,00260 x K TL/ton formülü ile hesaplanarak ödenir. Bu formüldeki (M), (K), (A) ve (Y), Poz 07,006/K daki gibidir. |  |

SURFACE ASPHALT COATING TECHNICAL SPECIFICATION

Road Preparation

1.

- The entire platform to be paved will be finely graded and the channels will be cleaned.

- The infratructure deficiencies other than the procument on the road shall be completed by the Coontracted Entity (Stabilize,shut, culvert etc.)

- The thickness of the screening material (o.13 mm) shall be at least 10 cm for each side of the road fort he first floor surface coating. After that, compression will start.

- After this process is done, sweeping will be done starting from middle of the road and the platform will be cleaned from all kinds of foreign material.

2.

- Priming should be done in dry, no precipitation and hot weather, the temperature of the shade should be 15° C. Even if the air temperature is more than 25° C, no priming will be done if the surface moisture is more then 1%.

- The spraying temperature should be adjusted to the ambient temperature.

3.

- Lining should should not be done in cold weather, on wet floors and in chance of rain in weather conditions.

- If the temperature of the shade is below 10° C, the bitumen should not be thrown.

- The bitumen application temperature should be 165° C.

- Before the primed road is covered, precautions must be taken on the road in terms of work and traffic safety and road traffic should be closed.

4.

- Gravel; 25-19 mm fort he first coat, 16-13 mm fort he second coat and free from dust.

- In the agreegate where the gravel can be obtained; dust vegetative substances or any other harmfull substances. There will be no harmful subtances such as a axcess clay sils, coal, wood or wood fragments in the proportion of 2% by weight.

5.

- For the first layer surface coating, 2.0 kg of bitumen(primer) and 1.8 kg asphalt shail be added to each m2 of the road.

- For the second layer surface coating, 1.7 kg of ashalt will be added per m2.

- For the first layer surface coating, the road will pour 0.02 m3 type A gravel per m2.

- For the second layer surface coating, the road will pour 0.0017 m3 type B gravel per m2.

- The cylinder will start after the grave has been laid. The cylindrical tire is to be made with a wheeled cylinder.

- The miling will be done along the road, starting from the edge, approaching the middle of the road.

- The sealing must continue until the agreegate granules are placed in the binder bituminous material.

- The binder bituminous material should not be cured after hadernning. Otherwise, the bond between the bituminous material and the agreegate granules will deteriorate.

6.

- Sieves and rocks to be used shall be subjected to sieve analysis and used after approval of the test results by the administration. It is compulsory that the roadworthhiness certificate is submitted to the administration before the material is withdrawn.

- The gravel to be used will be crushed stone or cracked gravel and will consisit of clean, rough, firm and durable grenaries. In agreegate, soft and dull parts clay, organic and other harmful substances will not be free or agreegated. If it is of volcanic origin, the chemical additive which increases the peel strength will be used. This rate will be determined by peeling test. The peel strength will be at least 50%.

7. Asphalt cements to be used in surface asphalt coating will be penetration in accordance with TS 1081 (AC 160-220) this will be delivered as a confirmation document from the licensed laboratory or from official institutions or the refinery exit bill linked tu TÜPRAŞ and sealed tanker and scales.

8. The primer material will be accordance with TS 1083 (MC 30). The quality ıf the viscosity will be checked according to the principles of the Article 7 of the technical specification and the refirnery output bill connected to TÜPRAŞ will be delivered to the sealed tanker and scales. The spraying temperature will be 50° C.

9.Before the second floor asphalt coating is done , maintenance , repair and patch work must be done by the administration.

Machines required for surface asphalt coating ;

1.	Distiributor -2 psc.

2.	Iron Bandage Cylinder – 1 psc.

3.	Rubber- tired roller – 1 psc.

4.	Loader – 2 psc.

5.	Grader 2 psc.

6.	water-truck – 1 pc.

7.	Gritter (Damtrack) – 5 psc.

8.	Broom – 2 psc.

9.	Dump Truck – 5 psc.

10.	Carrier Roley Tank – 1 psc.

11.	Asphalt Pump – 1 psc.

12.	Heating Rolley Tank – 1 psc.

13.	Asphalt, Asphalt Boiler– 2 psc.

14.	Figure Machine – 1 psc.

15.	Water pump – 1 psc Depending on the volume of work , it is also necessary to increase the number of teams and machine parks.

10. The current road between Sarpdere Village – Dupnisa is between 5.00 m and 7.50 m. The  road to be constructucted within the scope of the project will be a total lenght of 8 m surface coating including a 6 meter motorway snd 2 meter bicycle path.

SATHİ ASFALT KAPLAMA YAPIM TEKNİK ŞARTNAMESİ

Yolun Hazırlanması

1.	a)	Asfaltlanacak yolun tamamının platformu greyderle ince reglaj yapılacak, kanalları temizlenecektir.

b)	Yoldaki ihale dışında kalan alt yapı eksiklikleri (stabilize,büz, menfez vs.) idare tarafından tamamlanacaktır.

c)	1.kat sathi kaplama için yolun her noktasında elekaltı malzemenin (0,13 mm) kalınlığı en az 10 cm olacaktır. Bundan sonra sıkıştırma işlemine başlanacaktır.

d)	Bu işlemler yapıldıktan sonra yolun ortasından başlamak suretiyle süpürme işlemi yapılacak, platform her türlü yabancı maddeden temizlenecektir.

2.	a)	Astarlama kuru, yağışsız ve sıcak havalarda yapılacak, gölgede sıcaklık 15 0C olmalıdır. Hava sıcaklığı 25 0C fazla olsa bile, yüzey rutubeti %1’ den fazla ise astarlama yapılmayacaktır.

b)	Püskürtme sıcaklığı astar malzemeleri çevre sıcaklığına uygun yapılmalıdır.

3.	a)	Astarlama soğuk havalarda, ıslak zeminlerde ve yağmur ihtimali olan havalarda yapılmamalıdır.

b)	Gölgede ısı 10 0C nin altında ise bitüm atılmamalıdır.

c)	Bitümün tatbikat ısı derecesi 165 0C olmalıdır.

d)	Astarlanmış yola bitüm atılmadan önce, yolda iş ve trafik güvenliği açısından önlemler alınmalı, yol araç trafiğine kapatılmalıdır.

4.	a)	Mıcır; birinci kat için 25-19 mm, ikinci kat için 16-13 mm ebatlarında ve tozdan arındırılmış olmalıdır.

b)	Mıcır elde edilecek agreganın içinde; toz, nebati madde veya herhangi başka bir zararlı madde bulunmayacaktır. Agreganın bünyesinde ağırlık oranı olarak %2’ den fazla kil toprakları, kömür, odun veya tahta parçaları gibi zararlı maddeler bulunmayacaktır.

5.	a)	Birinci kat sathi kaplama için yolun her m2 sine 2.0 kg bitüm (astar), 1.8 kg asfalt atılacaktır.

b)	İkinci kat sathi kaplama için yolun her m2 sine 1.7 kg asfalt atılacaktır.

c)	Birinci kat sathi kaplama için yolun her m2 sine 0,022 m3 A  tipi mıcır dökülecektir.

d)	İkinci kat sathi kaplama için yolun her m2 sine 0,017 m3 B  tipi mıcır dökülecektir.

e)	Mıcırın serilmesinden sonra silindiraj başlayacaktır. Silindiraj lastik tekerlekli silindirle yapılacaktır.

f)	Silindiraj yol boyunca, kenardan başlayarak, yolun ortasına doğru yaklaşılarak yapılacaktır.

g)	Silindiraj agrega tanelerinin bağlayıcı bitümlü malzemeye yerleşinceye kadar devam etmelidir.

h)	Bağlayıcı bitümlü malzeme sertleştikten sonra silindiraj yapılmamalıdır. Aksi halde bitümlü malzeme ile agrega tanelerinin bağlantısı bozulur.

6.	a)	Kullanılacak olan elekatı ve mıcırlar, elek analizine tabi tutularak, deney sonuçları idare tarafından onaylandıktan sonra kullandırılacaktır. Karayolları uygunluk belgesinin malzeme çekilmeden önce idareye ibraz edilmesi zorunludur.

b)	Kullanılacak olan mıcır, kırmataş veya kırılmış çakıl olacak ve temiz, pürüzlü, sağlam ve dayanıklı danelerden oluşacaktır. Agrega içinde yumuşak ve dayanıksız parçalar, kil, organik ve diğer zararlı maddeler serbest veya agregayı sarmış halde bulunmayacaktır. Eğer volkanik kökenli olursa soyulma mukavemetini artırıcı kimyasal katkı maddesi kullanılacaktır. Bunun oranı soyulma deneyi yapılarak belirlenecektir. Soyulma mukavemeti minimum % 50 olacaktır.

7.	Sathi asfalt kaplama yapımında kullanılacak olan asfalt çimentoları TS 1081’ e uygun (AC 160-220) penetrasyonlu olacaktır. Bunu doğrulayıcı belge olarak lisanslı laboratuardan veya resmi kurumlardan alınmış rapor veya TÜPRAŞ’ a bağlı rafineri çıkış faturaları ile mühürlü tanker ve tartı fişi idareye teslim edilecektir.

8.	Astar malzemesi TS 1083’ e uygun (MC 30) olacaktır. Viskozitesi bu teknik şartnamenin 7. maddesinin esaslarına göre kalite kontrolü yapılacak ve TÜPRAŞ’ a bağlı rafineri çıkış faturaları ile mühürlü tanker ve tartı fişi idareye teslim edilecektir. Püskürtme sıcaklığı 50 0C olacaktır.

9.	2.kat sathi asfalt kaplama yapılmadan önce idare tarafından yolun bakım, onarım ve yama işinin yapılması gerekmektedir.

Sathi Asfalt Kaplama Yapımı İçin Gerekli Olan Makineler

1.	Distiribitör – 2 ad.

2.	Demir Bandajlı Silindir – 1 ad.

3.	Lastik Tekerlekli Silindir – 1 ad.

4.	Yükleyici – 2 ad.

5.	Greyder – 2 ad.

6.	Arazöz – 1 ad.

7.	Mıcır Serici (Damtrak) – 5 ad.

8.	Süpürge – 2 ad.

9. 	Damperli Kamyon  – 5 ad.

10.	Taşıyıcı Roley Tank – 1 ad.

11.	Isıtıcılı Roley Tank – 1 ad.

12.   Asfalt Pompası – 1 ad.

13.   Astar, Asfalt Kazanı – 2 ad.

14.   Figüre Makinesi – 1 ad.

15.   Su Pompası – 1 ad.

İş hacmine göre, ekip sayısı ve makine parkının  da artırılması zorunludur.

10.  Mevcut Sarpdere köyü – Dupnisa maharası arası yol  5.00 m ile 7.50 m arasında değişmektedir.Proje kapsamında yapılacak olan yolun 6 metresi araç yolu ve 2 metresi de bisiklet yolu olmak üzere toplam 8 metre sathi kaplama genişliği olacaktır.